| ADS PLUS Streamlining Arid West Wetland Delineation Through Mobile AI | USACE Regulatory Program Arid West Region June 2026 |
| --- | --- |

THE PROBLEM — Administrative Overhead in a Multi-Tool Regulatory Workflow

| USACE regulatory staff conducting wetland delineations in the Arid West currently navigate three separate tools across two environments: Survey 123 for field data entry, the National Regulatory Viewer (NRV) for transfer, and the ADS Excel workbook for inference and form output. This fragmented pipeline creates compounding inefficiencies that directly impact mission delivery and data quality. ▸ Post-field re-entry burden: Data from Survey 123 must be manually transferred through the NRV into the ADS before any inference is available — requiring return to the office after every field session. ▸ Silent data contradictions: Indicator selections made in Survey 123 can conflict with ADS auto-inference on the same data. No flag, no alert, no reconciliation step exists on the final ENG Form 6116-1. ▸ Formula fragility: The ADS Excel formula network can be corrupted by routine operations (copy-paste, data entry) — producing incorrect determinations with no user-visible warning. ▸ Zero field guidance: Neither tool explains why an indicator fires or fails. Delineators work without a real-time QC or reference system in the field. ▸ No camera integration: Soil color requires a physical Munsell chart, carried separately, read subjectively under variable lighting — a significant source of inter-observer error. | CURRENT WORKFLOW FIELD Survey 123 (Esri) ↳ Mobile entry — no inference, no QC, no guidance ↓ OFFICE NRV Upload ↳ Manual post-field data transfer — connectivity required ↓ OFFICE ADS Excel Workbook ↳ Inference & PDF — laptop required, formula-fragile ↓ OUTPUT ENG Form 6116-1 via NRV ↳ Risk of Survey 123 / ADS data contradiction on final form |
| --- | --- |

THE AI-POWERED SOLUTION — ADS Plus

ADS Plus is a browser-based, offline-capable application that consolidates the entire three-factor wetland delineation workflow — vegetation, hydric soils, and hydrology — into a single HTML file running on any smartphone, tablet, or laptop with no installation, server, or internet connection required.

| ▸ AI Camera — Color Correction Matrix (CCM): A printed reference card with known Lab values enables real-time least-squares regression to derive a per-frame 3×3 Color Correction Matrix, correcting camera spectral response for any field lighting. Replaces subjective physical Munsell chart reading. ▸ AI Camera — Automated Redox Detection (SoilAnalyzeCapture): K-means++ clustering (K=6) partitions the full color distribution of a ped face. Clusters are matched to Munsell chips via CIE76 ΔE. Concentrations classified by USDA Table A1 contrast; depletions by absolute Munsell chroma. Outputs matrix color, feature type, Munsell notation, and % area from a single photograph — a capability with no equivalent in any existing USACE field tool. ▸ Automated Indicator Inference with Reason Strings: Evaluates all applicable hydric soil and hydrology indicators from entered data, returning plain-language explanations of exactly which horizon at which depth with which measurements triggered each indicator — supporting both documentation and field training. ▸ Data Consistency Enforcement: Real-time warnings surface when manually selected indicators conflict with entered data, LRR restrictions, above-layer chroma rules, or problematic indicator conditions — preventing the silent contradictions inherent in the multi-tool pipeline. | ▸ In-App ENG Form 6116-1 PDF Generation: A filled, print-ready data sheet generated directly within the application. Eliminates the NRV workflow step for form output entirely. ▸ Complete Offline Operation: Single HTML file. No installation, no App Store, no server, no internet connection after initial download. All data stored locally via browser localStorage. ▸ Full JSON State Export: Every horizon, selection, inference result, GPS coordinate, and remark exportable to a structured JSON archive — providing a complete, auditable, peer-reviewable data record. ▸ Single-Application Workflow: Replaces Survey 123 → NRV → ADS → NRV entirely. Field data entry through PDF output — one app, one session, one device, zero post-field transfer. |
| --- | --- |

EXPECTED IMPACT & IMPLEMENTATION REQUIREMENTS

| EXPECTED IMPACT • Eliminates multi-step post-field pipeline; estimated 30–60 min saved per delineation point • Removes structural risk of cross-tool data contradictions on the ENG Form 6116-1 • Increases technical accuracy: implements current NRCS Field Indicators 9.3 depth thresholds for F3 and F8, correcting known formula holdovers in the ADS • Context-aware inference explanations accelerate delineator training • No hardware cost beyond a government-issue smartphone or tablet | IMPLEMENTATION REQUIREMENTS • Platform: Any modern smartphone, tablet, or laptop (iOS, Android, Windows, macOS) • Distribution: Single HTML file — no App Store, no installation, no configuration • Network: None required for operation; one-time file transfer for initial deployment • Camera (AI modules): Standard rear-facing smartphone camera with autofocus • Reference Card: Printed on a standard color printer; template included in app • Approval Authority: Formal adoption requires USACE Regulatory Program and HQ USACE coordination |
| --- | --- |

CYBERSECURITY, RECORDS MANAGEMENT & POLICY COMPLIANCE

▸ Cybersecurity (AR 25-2): All data remains on-device via browser localStorage — no data transmitted to any server, cloud service, or external system. No ATO required for local browser execution.

▸ Records Management (AR 25-400-2): Full JSON export provides a timestamped, auditable record of all entered data, selections, and calculated results, consistent with Army records management requirements.

▸ Privacy (DoD Privacy Program): No PII collected or transmitted. GPS coordinates stored locally; included only in user-initiated exports under delineator control.

▸ AI Governance (DoD AI Strategy / EO 13960): AI capabilities — CCM color correction, k-means clustering, indicator inference — are transparent, explainable, and human-supervised. The delineator reviews all results and retains final determination authority at all times.

▸ Authoritative Basis: 1987 USACE Wetlands Delineation Manual; Arid West Regional Supplement (2008); NRCS Field Indicators of Hydric Soils Version 9.3 (Feb 2026 errata); ENG Form 6116-1 (Sep 2024).

ADS Plus v15  |  USACE Arid West Regulatory Program  |  Innovation Proposal  |  June 2026  |  UNCLASSIFIED